Geschwister-Scholl-Str.6A	  Tel. +49 (0) 6146 – 906 961 2    www.tks-solutions.com

65239 Hochheim-Germany	  Fax +49 (0) 6146 – 906 961 3	 info@ tks-solutions.com

###### For Sale:  Second Hand Steam Generators

Offer Ref.:  SB-10.26

(2) Steam Generator 32 t/h

Coal / LDO firing

Historical data

| Year of manuf. | 1958 / 1959 |
| --- | --- |
| Shut down | 2006 1991 installation of LDO Burners |
| Total running hrs | Approx.: 300,000 hrs |
| Availability | At once |
| Firing | Coal / LDO |
| Serviced | Continuously, in good shape Last check of piping thickness showed surprisingly very low wear and tear and all measured pipes at critical locations will stay for further approx. 150,000 hrs. |

Design						         Watertube boiler with membrane walls

Manufacturer                                          	Sulzer

Steam Rating                                    	32 t/h

Operation pressure                                	65 bara, operated at 55 bara

Steam Temperature                               	475 °C

Fuel used		                           	hard coal

Travelling Grate				(w x l) 3.6 x 6.6 m, 23,76 m2

Heating surface				1,296 m2

1 boiler drum, 2 x super heaters, 1 x Eco, Air Preaheater

Scope of supply

Boiler with all auxiliaries, pipes, pumps, without ash extraction belts, etc.

Offer Ref. STG-B-20.21

Steam Turbine Generator Set

Backpressure Type, 6 MW

Total running hrs				 Approx. 250,000 hrs

Year of Manuf.				1965

Condition					 Installed, available immediately

Shut down	2005

Turbine

Manuf.					Siemens / Germany

Rating						6.4 MW

Rotating Speed				8.000 rpm

Inlet steam pressure			57-65 bara

Inlet steam temperature 			472°C

Bleeding 					Wheel Chamber

Back Pressure 				2.2 – 3.2 bara

Main Revisions				1997 Turbine approx. 50,000 hrs before shut

Finding of Material test (Surface Crack

Examinations) via Dye Penetrant Test:

No indications Gefügeveränderung

2002 Alternator approx. 30,000 hrs before shut down, Stator rewinded, Protection and Synchronisation new.

Gear

8,000 / 1,500 rpm

Generator

Manuf.					Siemens Schuckert

Apparent power				8 MVA

Speed						1,500 rpm

Voltage					3.3 kV, 1,400 A, 50 Hz

Cos Fi						0.8

Excitation					dynamic with excitation machine

Cooling					Water / Air

Weights					Stator: 13 t, rotor: 7 t

Scope of Supply

Complete steam turbine with generator and auxiliaries as arranged mainly

steam turbine, oil system, alternator-oil & cooling system, filter, coolers, oil tank, inter connecting pipes, electrical installation, instrumentation, control cabinets spare parts as far as available, synchronizing, AVR, manuals, drawings and documentation.